微  波  板

应    用

一、自动门激活							二、车、房入侵报警

三、碰撞预告								四、交通、道路监控

微波探测传感器简介

微波探测传感器应用Doppler Radar原理，发射一个低功率微波并接受物体反射过来的能量。一旦物体的运动被其探测到。发射频率就被反射回的微波频率所替代，替代的微波与发射的微波混合在一起，结果一个低频率的电压从传感器输出。原理图如下：

微波探测传感器的特性：

- 非接触式。

- 周围环境：不受热、嗓音、湿度、气流、尘埃等影响，适合恶劣环境。

- 抗干扰。

- 安全。

- 宽范围。探测范围超过１５米。更宽的范围亦可能。

微波运动传感器

特性:

如下参数是在5VDC， CW 工作状态， 12KΩ负载， +25℃下测定。

| 参数 | 备注 | 最小值 | 典型值 | 最大值 | 单位 |
| --- | --- | --- | --- | --- | --- |
| 频率 | 1 | 10．520 | 10．525 | 10．530 | GHz |
| 辐射功率 | 1 | 12 | 15 | 20 | dBm |
| Spurious Emission @ 3m | 1 |  |  | 25 | μV/m |
| Settling Time |  |  | 3 | 6 | μSec |
| Received Signal Strength | 2 | 100 |  | 250 | μVp-p |
| Noise | 3 |  |  | 5 | μVrms |
| Supply Voltage |  | 4.75 | 5.00 | 5.25 | VDC |
| Current Consumption |  |  | 30 | 40 | mA |
| Pulse Repetition Frequency | 4 |  | 2 | 3 | KHz |
| Pulse Width | 4 | 10 |  |  | μSec |
| Operating Temp |  | -15 |  | 55 | ℃ |

Note1: The radiated emissions is designed to meet FCC rules.

Note2:The Received Signal Strength(RSS) is measured at the total 2

Ways path loss of 93dB.

Note3: The noise voltages are measured from 10Hz to 100Hz at the

Output port, inside an Anechoic chamber.

Note4: Pulse operation

特点及应用：低功耗//； CW 或Pulse 工作；长探测范围；

用于：微波红外运动探测；自动门；灯的控制；速度测试。

欢迎来电洽购我司产品或索取免费详细资料、设计选型指南和光盘 、样品；产品繁多未能尽录，欢迎来电查询。

中国传感器科技信息网：HTTP：//WWW.SENSOR-IC.COM/

工控安防网：HTTP：//WWW.PC-PS.NET/

消费电子专用电路网：HTTP://WWW.SUNSTARE.COM/

E-MAIL： xjr5@163.com  szss20@163.com

MSN: suns8888@hotmail.com

QQ:195847376

地址： 深圳市福田区福华路福庆街鸿图大厦1602室

电话：0755-83778810 83394033 83376282 83387030  83387016

传真：0755-83376182  83338339   邮编：518033  手机：(0)13902971329

深圳展销部：深圳华强北路赛格电子市场2583号  TEL/FAX：0755-83665529  13823648918

北京分公司：北京海淀区知春路132号中发电子大厦3097号

TEL：010-81159046  82615020  13823791822  FAX：010-62543996

上海分公司：上海市北京东路668号上海賽格电子市场地下一层D25号

TEL：021-28311762  56703037  13823676822  FAX：021-56703037

西安分公司：西安高新开发区20所(中国电子科技集团导航技术研究所)

西安劳动南路88号电子商城二楼D23号

TEL：029-81022619  13072977981  FAX:029-88789382

成都：TEL:(0)13717066236
技术支持：0755-83394033 13501568376